# Revision of Section 213

# Landscape Boulder (Special)

## Revise Section 213 of the Standard Specifications for this project as follows:

Subsection 213.01 shall include the following:

This work consists of furnishing and placing boulders in accordance with these specifications and in conformity with the details shown on the plans, or as directed.

Subsection 213.02 materials shall include the following:

(h)	Landscape Boulders (Special) Landscape boulders shall consist of hard, dense, sound fractured rock from the roadway excavation, subject to approval.  Thin slab type stones and flaking rock will not be accepted.  The boulders shall be spherical four foot to six-foot diameter, unless otherwise approved. Chose one of the following:

[Obtain boulders developed on the project site during blasting operations.]

[Locate suitable boulders and obtain the Engineer’s approval of the selected boulders prior to transport. Deliver boulders to a stockpile area designated by the Engineer.] ☖

Subsection 213.04 shall include the following:

The pay item Landscape Boulder (Special) will be measured by the number delivered to and stockpiled.

Subsection 213.05 shall include the following:

| Pay Item | Pay Unit |
| --- | --- |
| Landscape Boulder (Special) | Each |

The pay item Landscape Boulder (Special) will include locating suitable boulders, obtaining Engineer approval, moving boulders from project site to the stockpile area and unloading at the stockpile area.

Instructions to Designers: (Please delete these instructions before including in the project.)

☖ Work with the design engineer to determine if suitable boulders exist on site or select the second option of importing boulders.