# Revision of Section 506

# Riprap (Special)

Revise Section 506 of the Standard Specifications for this project as follows:

Subsection 506.01 shall include the following:

Riprap (Special) consists of placing riprap in the new channel.

Subsection 506.02 shall include the following:

Channel liner riprap shall consist of rounded native rock or cobble of the approximate size depicted on the plans or as directed. Concrete or artificial materials will not be permitted.

Subsection 506.03 shall include the following:

Excavate to a depth for placement to match the required size of the riprap. Use equipment to compact stones into the channel until even with top elevations of cobble.

Subsection 506.05 shall include the following:

Pay under:

Pay Item					Pay Unit

Riprap (Special)			Cubic yard

Include Structure Excavation in the work cost; do not pay for it separately.