# Section 220

# Lawn Seeding

Description

- This work consists of applying and preparing topsoil by incorporating compost (mechanically applied), furnishing and installing seed and establishing a lawn. This specification is primarily for repairs to existing irrigated lawns in urban areas for medians, and areas adjacent to sidewalks, ramps, and trails. Work includes an establishment period during which irrigation of seeded areas, at the rate, frequency and duration needed to establish a uniform coverage of lawn grass.  Mowing, weeding, and fertilization are also part of the establishment period.

- Definitions.

Bulk Material: See definition Section 212.02

Pesticide: See definition Section 217.02.

Pesticide Applicator. See definition Section 217.02.

## Materials

- Material Requirements

        - Topsoil. Furnish either salvaged Topsoil (Onsite) or Topsoil (Imported) that meets the properties and requirements of Section 207.03.

        - Compost. Furnish Compost that meets the properties and requirements of Subsection 212.03.

        - Lawn Seeding. Furnish lawn seed mixes in amounts and species as per Contract. Repairs shall match species of existing lawn. All seeding shall be mixed by a seed supplier for the specified species and rate. Seed shall be guaranteed for purity and germination and supplied on a Pure Live Seed (PLS) basis. Seed and seed labels shall conform to the Colorado Seed Act.

        - Organic Fertilizer. Furnish an organic fertilizer with 50 percent slow-release nitrogen. Fertilizer shall be in sealed containers as a dry free flowing granular pellet form suitable for application by a fertilizer spreader.

        - Mycorrhizae.  Furnish Mycorrhizae that meets the properties and requirements of Subsection 212.03 for the application type.

Table 220-1

Compost and Organic Fertilizer Application Rates

| Seed Type | Compost (cy/acre) | 20-5-10 Organic Fertilizer (lb/acre) 50% Nitrogen slow release | Mycorrhizae (lb/acre) |
| --- | --- | --- | --- |
| Drought Tolerant Seed Blends or Kentucky Blue Grass Seed | 175 | 220 | 0 |
| Native Grass Seed | 44 | 30 | 60 |

(e) Permanent Stabilization.

(1) Erosion Control Blanket: Blankets shall be 100% biodegradable and shall be composed of natural fibers mechanically bound together to form a continuous netting matrix.

(2) Dry Broadcast Mulch: Mulch shall consist of a dry granular mulch applied with a drop or broadcast spreader or by hand. The mulch shall consist of both wood and cellulose fibers, dyed green, biodegradable, and should not inhibit plant growth.

(f) Equipment. Furnish all equipment clean and free of plant material, noxious weeds and contaminated soil.

- Compost Equipment. The Contractor shall furnish equipment to complete tillage of compost.

- Broadcast Seeding Equipment. The Contractor shall furnish broadcast seeding equipment with a metering device.

- Broadcast Spreader.  The Contractor shall furnish an agricultural spreader to complete application of organic fertilizer and dry broadcast mulch.

- Rototiller, disc and harrow, field cultivator, vibra-shank, or other method suitable to site conditions.

- Lawn Roller.

## Construction Requirements

- Submittals.

Include the lawn seeding 60-day establishment period as milestones in the approved Baseline Schedule. Lawn seeding shall occur within the Lawn Seeding Window in Table 220-2.

Approval Process. Compost, Organic Fertilizer, lawn seed, and dry broadcast mulch are subject to acceptance in accordance with Section 106.012:

- Lawn Seed. Submit a seed vendor’s packing slip confirming the Contractor has not been in possession of the seed for more than 30 calendar days. The seed tag species, quantities, weights must match the Plans.

- Compost

- Producer’s name, address, telephone number, email, and web address

- Documentation that the compost Manufacturer is a participating member of in the U.S. Composting Council’s Seal of Testing Assurance Program

- Producer’s technical data sheet confirming all required test methods are met.  Testing results shall include data within 90 days of material acceptance and include 1 year of data

- Organic Fertilizer

- Manufacturer’s name, address, telephone number, email, and web address

- Product data sheet

- Manufacturer’s chemical/nutrient analysis that meets properties in Table 212-5 for the requirements shown on plans

- Mycorrhizae. Submit a copy of the mycorrhizae manufacturer's product data sheet showing:

- Manufacturer’s name, address, telephone number, email, and web address

- Product data sheet

- Species present and number of propagules per pound

- Production date

- Landscape Stabilization Method. Submit manufacturer’s cut sheet for approval.

(c)	Pesticide Applicator. Applicators shall be a Colorado licensed commercial applicator and meet all requirements of Section 217.

(d) Submit an Operations and Maintenance/watering schedule to be implemented during lawn seeding establishment. Include directions for irrigation, aeration, mowing, fertilizing, and applying pesticides as required for continued maintenance for a full growing season and dormant period.

- Site Pre-vegetation Conference.  Per Section 207.

Table 220-3

Lawn Seeding Seasons

| Lawn Seed Type | Irrigated Areas Only | Non-Irrigated Areas |
| --- | --- | --- |
| Cool Season Grass Lawns - Kentucky Bluegrass, Fine or Tall Fescue and Blends | March 15 to Sept 1 | N/A |
| Warm Season Grass Lawns - Buffalograss, Blue Grama or Cynodon | April 1 to July 15 | N/A |

The Engineer, coordinating with the Region Environmental Manager, will approve seeding activities outside the seeding seasons only when requested in writing. Irrigation systems shall be properly functioning before lawn seeding activities and be active for 21 calendar days after seeding for lawn seeding establishment.

If the Engineer directs seeding outside the seasons listed above, the cost to repair areas that fail to produce species will be paid for by the Department.

- Delivery, Storage, and Handling

- Seed. Seed shall not be in the Contractor’s possession for more than 30 calendar days from the date on the seed vendor's packing slip.

- Bulk Materials:

- Do not dump or store bulk materials within 15 feet, or as approved by the Project Engineer, of structures, utilities, walkways and pavements, or on existing grass areas or plants.

- Compost.  Shall be stored away from other materials. Compost shall not be contaminated by on-site construction material.

- Material will be inspected upon arrival at the project site. The Project Manager will reject any opened or unacceptable materials as described above.

- Immediately remove unacceptable material from the job site.

- Site Preparation, Topsoil Amendments, and Seeding

General. Complete seeding within four working days of application of topsoil. All amendments and seed shall be applied based on the seeding method and rates as Contract.

Topsoil preparation and seeding installation shall be conducted under favorable weather conditions as approved by the Engineer. Soil preparation and seeding installation shall not occur when the topsoil is in a frozen, excessively wet, or in a dry-crusted state. Topsoil shall be able to make a ball in the palm of the hand and be moist enough to stick together and then break apart easily, yet not be so dry and hard that it either does not stick together or does not break apart easily. Dry hard soils should be moisture conditioned with a fine spray by nozzles or spray bars, in a manner to avoid erosion. Suspend topsoil placement, grading, and tilling operations during periods of excessive soil moisture until the soil moisture reaches levels determined acceptable by the Engineer.

Remove and dispose of all surface debris offsite including rocks, concrete, asphalt, tree branches, and tree stumps greater than 1 inch in diameter in any direction from the topsoil surface.

Ensure all final grades and slopes are maintained, allowing for positive drainage in all areas to be seeded.

Notify the Engineer a minimum of 3 working days prior to the start of seeding activities.

- Weed Control. All vegetation in the area to be seeded shall be removed before reapplication of topsoil, incorporating topsoil amendments, and seeding. All noxious and invasive weeds shall be mowed, bagged, and hauled off-site.  Chemically treat the remaining living vegetative shoots and stems by a licensed applicator.

- Topsoil Spreading. Spread topsoil from on-site stockpiles or imported source over prepared grades. Spread topsoil to a depth, as shown in the plans, over all areas to be seeded. Grade to a smooth surface.

- Topsoil Amendment. Limit amending topsoil to areas that can be seeded within 1 week. Do not proceed with Topsoil Amendments until the Engineer has approved in writing the specified and completed weed control measures and topsoil amendment.

- Apply compost uniformly over the topsoil and till to a minimum depth of 6 inches until uniformly incorporated into the soil.

- Remove all sticks, stones, debris, clods, asphalt, concrete, and other material more than 1 inch in any dimension from the topsoil surface and dispose of it properly offsite. Rake by hand or mechanically to a smooth, even surface.

- Maintain positive drainage at all times.

- Finish grade to be 1 inch below the edge of pavement or curb prior to seeding.

- Repair areas eroded or otherwise disturbed after fine grading and prior to lawn seeding. Depressions or variances from a smooth grade shall be corrected.

- Seeding and Stabilization.

Seeding shall not occur until the irrigation system is functioning as designed.

  - No existing vegetation shall be present on areas to be seeded.

  - Using a broadcast spreader, uniformly apply lawn seed at the rate specified in the plans.

  - Rake seed to ensure seed to soil contact.

  - Install dry broadcast mulch, unless erosion control blanket is specified in the plans.

  - Immediately after seeding and mulching, the Contractor shall lightly water the site, until the soil is moist to 1-inch depth.

  - Contractor shall set the irrigation program to apply water in light and frequent applications. Contractor shall repair and re-seed areas that have eroded or are not a uniform stand of grass.

- Establishment

(a)	The lawn seeding establishment period shall begin immediately after each area is seeded and continue for 60 calendar days.  During this time, the Contractor is responsible for watering, mowing, weeding, fertilizing, and all related work as necessary to ensure that seeded areas germinate, establish, and are in a vigorous growing condition. Furnish all supervision, labor, material and equipment to seed and maintain seeded areas.

(1)	Watering. Seeded areas shall be watered with a fine spray within 24 hours of seeding. Water seed twice daily or as necessary to maintain soil moisture but not completely saturate seed with water, within the top 2 inches of soil. Continue watering until grass has grown to a height of 3 inches. It shall be the responsibility of the Contractor to ensure proper operation of the existing irrigation system and to program a temporary watering schedule for grass establishment.  If there is no irrigation system present, the Contractor shall provide a means to irrigate for the grass establishment period.

2)	Reseeding.  It is expected that areas seeded will receive consistent irrigation, germinate, and begin growing within 30 days.  30 days after lawn seeding, the growth will be evaluated and determined if and where the lawn needs to be reseeded at no cost to the Department.

(3) Mowing and Trimming. When grass reaches 3 1/2 inches in height, begin a weekly mowing program to maintain lawn grasses at 2 1/2 inches to 3 inches in height. Do not remove more than 1/3 the height of the grass blade in a single mowing. Do not mow when soil is wet. All clippings from adjacent paved areas shall be removed.

(4)	Fertilizing. Approximately 30 calendar days after installation and once the grass has established, apply an organic fertilizer as specified in Table 220-1 above.

(5)	Weed Control. Control weeds by mowing and manually pulling. Before application of herbicides, the Contractor shall check with local county, city, or town on environmental regulations prior to herbicide prior to use.

- Acceptance

At the end of the 60 day establishment period, the Engineer will perform a final walkthrough to ensure the establishment criteria, a healthy uniform stand of grass in vigorous growing condition with 80-90 percent coverage of specified lawn seed species, and lacking stressed, yellow or dying grass, or areas of visible soil greater than 1 foot in diameter have been met. Final Acceptance will be issued once lawn seeding has achieved acceptance criteria for the project. The Contractor shall obtain written approval from the Project Engineer and Maintaining Agency on agreement of condition of lawn seeding and submit to Project Engineer before to Final Acceptance of the lawn seeding.

## Method Of Measurement

The quantity of lawn seed will be the actual number of acres prepared, installed, and accepted.

## Basis of Payment

Payment will be made under:

| Pay Item | Pay Unit |
| --- | --- |
| Lawn Seeding | Acre |
| Lawn Seeding Establishment Add Period | Each |

Removal of all vegetation prior to applying lawn (seeding) will not be measured and paid for separately but shall be included in the work.

Lawn seeding includes topsoil preparation, compost, landscape stabilization mulching or blanketing, a 60-day establishment period that includes furnishing, applying fertilizer, watering, weeding, and mowing, to get a full healthy stand of grass. This will not be measured and paid for separately but shall be included in the work.

Areas that fail to establish and grow specified seed type shall be reprepared and seeded, and the landscape establishment period shall restart for an additional 60 days at no cost to the Department.

Calibrating, adjusting, or readjusting fertilizing equipment will not be measured and paid for separately but shall be included in the work.

Additional soil preparation to correct compaction or erosion after storm events that occur between topsoil placement and seeding activities will not be measured and paid for separately but shall be included in the work.

All operation and maintenance plans and watering schedules are required but included in the cost of the work.