Revise Section 207 of the Standard Specifications for this project as follows:

Replace the second and third sentences of Subsection 207.01 with the following:

The work consists of furnishing all labor, equipment, and materials for the salvaging, stockpiling, maintenance, and final placement of onsite rocky topsoil.

Revise Section 207.03 to remove the Table 207-2 Textural Class Analysis:

Table 207-2 does not apply for this Rocky Soils PSP.

Revise Subsection 207.04 Submittals with the following:

Remove the fourth bullet on the subsoil preparation method, as it does not apply to this Rocky Soils PSP.

Replace the sixth bullet with the following:

Removal of debris, concrete, and asphalt for placed topsoil.

Revise Subsection 207.08 by adding the following paragraphs, at the end:

Rocky, gravely, and sandy subsoils do not require decompaction. All other subsoils to receive seeding (native) as final stabilization shall be decompacted per 207 Subsoil and Topsoil.

All embankment material shall be compacted per Section 203.

Replace the last sentence of Subsection 207.10 with the following:

Remove all concrete, asphalt, and other construction debris from topsoil greater than 3 inches in any direction.  Retain native rock and woody vegetative debris with a diameter less than 3 inches within the topsoil matrix. Do not screen or remove native materials unless otherwise shown on the Plans.

Subsection 207.13 shall include the following:

The quantities of Topsoil (Rocky) shall be measured by the actual volume of material placed and accepted, based on the measured area and depth designated in the Contract, or through another method approved by the Project Engineer. Include the cost of removing and disposing of non-native debris (concrete, asphalt, steel) in the unit price for Topsoil (Rocky).

Subsection 207.14 shall include the following:

| Pay Item | Pay Unit |
| --- | --- |
| Topsoil (Rocky) | Cubic Yard |

Payment for Topsoil (Rocky) shall be full compensation for all work necessary to complete the item, including salvaging, stockpiling, maintenance of stockpiles, including trash and weed mitigation, and final placement of material. Removal of trash and debris before clearing and grubbing, and topsoil stockpiling will be measured and paid per Section 208.

Instructions to Designers: (Please delete these instructions before including them in the project.)

The intent of this specification is to retain the native rocky topsoil where appropriate, allowing for a visual cohesion to blend the project into the context of its surroundings. This PSP is to be used only for retaining native onsite rocky topsoil.

If reusing rocky topsoil for areas to be revegetated, coordinate with the design team to determine if it is appropriate.  Then determine if retaining duff during clearing and grubbing is appropriate for topsoil salvaging.

Add boulders to the landscape plans to visually blend with the existing landscape. Boulders may be anticipated to be excavated from onsite, per Section 203 Excavation and Embankment, or purchased.  Include Landscape Boulder pay items and quantities, and the M-standard for Landscape Boulders and indicate type, size, and locations in landscape plans.  All boulders must be placed out of the clear zone.

212 Amendments and Native Seeding pay items implemented with this PSP include hydroseeding. Biotic Soil Amendment (Hydraulically Applied) with mycorrhizae and organic fertilizer are appropriate amendment pay items to reduce mobilization costs.

213 Mulching. Hydromulching is the appropriate pay item for final stabilization mulching on this Rocky Topsoil PSP. Note that Sediment Control Blanket is not an appropriate control measure for stabilizing rocky topsoil. Additionally, straw mulch is not an appropriate erosion-control method for stabilizing rocky soils because the straw cannot be effectively crimped into the rocky substrate.  Alternatively, a PSP for pneumatically applied compost is available for locations where compost is available.

Additional guidance is available on the CDOT Landscape Architecture page here.